FDE Interpretation # 202605

Status:                                  Active                                                       Inactive

Date: 06-16-2026

Type of Document:            Technical Decision  		      Technical Recommendation

Approved by:                      FDE iTC Interpretations Team 	      FDE iTC

Affected Document(s): FDE AA cPP v2.0e, FDE EE cPP v2.0e, FDE AA SD v2.0e, FDE EE SD v2.0e,

Affected Section(s): FCS_CKM.1.1(a), FCS_CKM.1.1(b), References

Superseded Interpretation(s): RFI202602

Issue:

Feedback from NIAP on RFI202602 noted that a simple replace would not suffice to resolve this issue.  It must address other related references.

Resolution:

To fully address the out of date reference this Technical Decision will replace RFI202602:

The FDE AA cPP v2.0e and FDE EE cPP v2.0e will make the following changes:

For FCS_CKM.1(a) Cryptographic Key Generation (Asymmetric Keys), the SFR selections:

RSA schemes using cryptographic key sizes of [selection: 2048-bit, 3072-bit, 4096-bit] that meet the following: FIPS PUB 186-4, “Digital Signature Standard (DSS)”, Appendix B.3;

ECC schemes using “NIST curves” of [selection: P-256, P-384, P-521] that meet the following: FIPS PUB 186-4, “Digital Signature Standard (DSS)”, Appendix B.4;

Will now read:

RSA schemes using cryptographic key sizes of [selection: 2048-bit, 3072-bit, 4096-bit] that meet the following: FIPS PUB 186-5, “Digital Signature Standard (DSS)”, Appendix A.1;

ECC schemes using “NIST curves” of [selection: P-256, P-384, P-521] that meet the following: FIPS PUB 186-5, "Digital Signature Standard (DSS)", Appendix A.2;

The 4th paragraph of the application note:

For all schemes (RSA schemes, ECC schemes, FFC schemes), an RBG is needed to a) generate seeds for RSA and to b) generate private keys directly for ECC and FFC. So FCS_RBG_EXT.1 is used together with this SFR. A hash algorithm is also required when the key pair generation algorithm is selected based on either Appendix B.3.2 or B.3.5 of FIPS 186-4. So in such case, FCS_COP.1(d) is used together with this SFR.

Will now read:

For all schemes (RSA schemes, ECC schemes, FFC schemes), an RBG is needed to a) generate seeds for RSA and to b) generate private keys directly for ECC and FFC. So FCS_RBG_EXT.1 is used together with this SFR. A hash algorithm is also required when the key pair generation algorithm is selected based on Appendix A.1.2, A.1.4, and A.1.5 of FIPS 186-5. So in such case, FCS_COP.1(d) is used together with this SFR.

For FCS_CKM.1(b) Cryptographic Key Generation (Symmetric Keys), the SFR selections:

FIPS PUB 186-4, “Digital Signature Standard (DSS)”, Section 5.5, using PKCS #1 v2.1 Signature Schemes RSASSA-PSS and/or RSASSA-PKCS1-v1_5; ISO/IEC 9796-2, Digital signature scheme 2 or Digital Signature scheme 3, for RSA schemes

FIPS PUB 186-4, “Digital Signature Standard (DSS)”, Section 6 and Appendix D, 2 Implementing “NIST curves” [selection: P-256, P-384, P-521]; ISO/IEC 14888-3, 3 Section 6.4, for ECDSA schemes

Will now read:

FIPS PUB 186-5, “Digital Signature Standard (DSS)”, Section 5, using PKCS #1 v2.2 Signature Schemes RSASSA-PSS and/or RSASSA-PKCS1-v1_5; ISO/IEC 9796-2, Digital signature scheme 2 or Digital Signature scheme 3, for RSA schemes

FIPS PUB 186-5, “Digital Signature Standard (DSS)”; ISO/IEC 14888-3, 3 Section 6.4, for ECDSA schemes

Appendix H: References adds the following below line number 18:

Federal Information Processing Standard Publication (FIPS-PUB) 186-5, Digital Signature Standard (DSS), National Institute of Standards and Technology, February 2023.

The FDE AA SD v2.0e and FDE EE SD v2.0e will make the following changes:

The FCS_CKM.1(a) Cryptographic Key Generation (Asymmetric Keys) test activity line:

Key Generation for FIPS PUB 186-4 RSA Schemes

Will now read:

Key Generation for FIPS PUB 186-5 RSA Schemes

The FCS_CKM.1(a) Cryptographic Key Generation (Asymmetric Keys) test activity line:

FIPS 186-4 ECC Key Generation Test

Will now read:

FIPS 186-5 ECC Key Generation Test

The FCS_CKM.1(a) Cryptographic Key Generation (Asymmetric Keys) test activity line:

FIPS 186-4 Public Key Verification (PKV) Test

Will now read:

FIPS 186-5 Public Key Verification (PKV) Test

The FCS_COP.1(a) Cryptographic Operation (Signature Verification) test activity line:

ECDSA FIPS 186-4 Signature Verification Test

Will now read:

ECDSA FIPS 186-5 Signature Verification Test

Rationale:

The reference is out of date.

Further Action:

No further action is required.

Action by FDE iTC:

No further action is required.